IP Address Plan

VLAN Address Plan

| VLAN ID | VLAN Name | Purpose | Network Address | Subnet Mask | Default Gateway (HSRP VIP) |
| --- | --- | --- | --- | --- | --- |
| 10 | IT | IT Department | 172.16.10.0/26 | 255.255.255.192 | 172.16.10.1 |
| 20 | HR | HR Department | 172.16.20.0/27 | 255.255.255.224 | 172.16.20.1 |
| 30 | Finance | Finance Department | 172.16.30.0/26 | 255.255.255.192 | 172.16.30.1 |
| 40 | Sales | Sales Department | 172.16.40.0/25 | 255.255.255.128 | 172.16.40.1 |
| 50 | Engineering | Engineering Department | 172.16.50.0/24 | 255.255.255.0 | 172.16.50.1 |
| 60 | Guest | Guest Wireless Network | 172.16.60.0/25 | 255.255.255.128 | 172.16.60.1 |
| 90 | Management | Network Device Management | 172.16.90.0/27 | 255.255.255.224 | 172.16.90.1 |
| 100 | Server | Server Network | 172.16.100.0/24 | 255.255.255.0 | 172.16.100.1 |
| 999 | Unused Ports | Unused ports |  |  |  |

Transit and WAN Networks

| Network | Purpose | Subnet Mask |
| --- | --- | --- |
| 172.16.200.0/29 | Edge Router ↔ L3SW1 | 255.255.255.248 |
| 172.16.200.8/29 | Edge Router ↔ L3SW2 | 255.255.255.248 |
| 10.0.0.0/30 | Edge Router ↔ ISP Router | 255.255.255.252 |

Router Interfaces

| Device | Interface | IP Address | Connected To | Purpose |
| --- | --- | --- | --- | --- |
| ISP-R1 | Gi0/0 | 10.0.0.2/30 | Edge-R1 Gi0/0 | WAN Link |
| Edge-R1 | Gi0/0 | 10.0.0.1/30 | ISP-R1 Gi0/0 | WAN Link |
| Edge-R1 | Gi0/1 | 172.16.200.3/29 | L3SW1 Fa0/1 | Transit Link |
| Edge-R1 | Gi0/2 | 172.16.200.9/29 | L3SW2 Fa0/3 | Transit Link |

Core Switch Interfaces

| Device | Interface | IP Address | Purpose |
| --- | --- | --- | --- |
| L3SW1 | Fa0/1 | 172.16.200.1/29 | Transit Link to Edge Router |
| L3SW2 | Fa0/3 | 172.16.200.10/29 | Transit Link to Edge Router |

Layer 3 VLAN Interfaces (SVIs)

| VLAN | Virtual Gateway | L3SW1 | L3SW2 |
| --- | --- | --- | --- |
| VLAN10 | 172.16.10.1 | 172.16.10.4 | 172.16.10.3 |
| VLAN20 | 172.16.20.1 | 172.16.20.4 | 172.16.20.3 |
| VLAN30 | 172.16.30.1 | 172.16.30.4 | 172.16.30.3 |
| VLAN40 | 172.16.40.1 | 172.16.40.4 | 172.16.40.3 |
| VLAN50 | 172.16.50.1 | 172.16.50.4 | 172.16.50.3 |
| VLAN60 | 172.16.60.1 | 172.16.60.4 | 172.16.60.3 |
| VLAN90 | 172.16.90.1 | 172.16.90.4 | 172.16.90.5 |
| VLAN100 | 172.16.100.1 | 172.16.100.4 | 172.16.100.3 |

Management Interfaces

| Device | Interface | Management IP |
| --- | --- | --- |
| L2SW1 | VLAN90 | 172.16.90.2 |
| L2SW2 | VLAN90 | 172.16.90.3 |

Routing Protocol Summary

| Device | Routing Method | Router ID |
| --- | --- | --- |
| ISP-R1 | Static Routing | N/A |
| Edge-R1 | OSPF Area 0 + Static Default Route + NAT | 3.3.3.3 |
| L3SW1 | OSPF Area 0 | 1.1.1.1 |
| L3SW2 | OSPF Area 0 | 2.2.2.2 |

Static Routes

| Device | Destination | Next Hop | Purpose |
| --- | --- | --- | --- |
| Edge-R1 | 0.0.0.0/0 | 10.0.0.2 | Internet Default Route |
| ISP-R1 | 172.16.0.0/16 | 10.0.0.1 | Route to Enterprise Network |